4.6	Repeat change (no standard)

It is possible to change the drive ratio between the main shaft and the pattern drum by exchan­ ging the toothed belt disks on the repeat preset gear as follows:

| repeat | "'toothed belt disk |  | *length of toothed belt | *arrangement of tension rollers |  |
| --- | --- | --- | --- | --- | --- |
|  | A | B |  |  |  |
| 1: 24 | 24 | 36 | 720mm | 1 |  |
| 1 : 20 | 36 | 45 | 800mm | 1 | no standard |
| 1 : 18 | 32 | 36 | 720mm | 2 | no standard |
| 1 : 16 | 40 | 40 | 800mm | 1 |  |
| 1 : 14 | 32 | 28 | 720mm | 2 | no standard |
| 1 : 12 | 40 | 30 | 720mm | 2 |  |
| . 1 : 10 | 48 | 30 | 800mm | 1 |  |
| *For order numbers, pis. see spare parts catalogue. |  |  |  |  |  |

L--